| # Catapult, Leverpult, Slingshot |  | Intermediate-Advanced |
| --- | --- | --- |

| # Project Goal Students build one: the catapult, leverpult, or slingshot that can launch a cork at least 15 feet. # Design Variables - None for basic example. # Key Concepts Energy transformation is when one kind of energy turns into a different form. Structural strength is how much weight or force something can withstand. A hinge is a movable joint that allows something to move back and forth (for catapult and leverpult only). ## Prep - Build a basic example - Cut 4 ½-sticks per student (for catapult and leverpult) ## Notes Prepare just one project (catapult, leverpult, or slingshot), but use this Lesson Essentials for all three projects. |  | ## Materials Basic Example - 16-20 craft stick - 0-4 ½-stick - 8 cube - 3 cube w/ hole - 1 skewer - 1 short r.band - 1 cup - tape - hot glue - caution tape Advanced Idea 2-Direction catapult - ½ straw Double basket catapult Build a custom machine that can launch a cork 15 feet. |
| --- | --- | --- |

| Troubleshooting |  |
| --- | --- |
| If the catapult is… | Then… |
| Not launching the cork at least 15 feet | There is not enough energy stored in the rubber band The base is not being firmly held in place The cup is too loose |
| If the slingshot is… | - Then… |
| Not launching the cork at least 15 feet | There is not enough energy stored in the rubber band Aim the slingshot higher The sling is too big The sling itself is being grasped instead of the pull tab |

# Introduce the project demonstrate how it works (1 min)

# Explain how it works (1 min)

- The concepts of this intermediate project should be familiar to an intermediate student.

- Briefly remind students of how energy transformation is manifesting.

- Note that the frame is created from a truss-based design. This give the project structural strength so it can withstand the force of the stretched rubber band.

- For the catapult and leverpult, note where the hinge is and discuss how it works.

# Describe how to build the project (6 min)

- Intermediate students should be familiar with the stick-and-cube frame building technique.

- Divide the project into three parts:

  - The base

  - The support structure

  - The catapult arm/slingshot

- Verbally walk students through how to build each part [Ex: The base is a rectangle made of 2 half-sticks, 2 normal sticks, and four cubes at the corners].

- Use a project sheet as a visual aide.

# Show how to lauch a cork (1 min)

- Show step-by-step how to successfully launch a cork.

  - For catapult: Firmly hold the frame onto a surface. Load the cork, pull back on the very end of the catapult arm (not the cup), quickly let go to launch.

  - For leverpult: Load the cork, firmly hold the frame onto a surface, and then swiftly press down on the other end of the catapult arm (avoid striking the lever too hard).

  - For slingshot: Grip the front of the base from underneath, load the cork, pinch the sling with two fingers, pull back and allow the sling to slip between your fingers.