| # Hydraulic Box |  | Intermediate - Advanced |
| --- | --- | --- |

| # Project Goal Students build a box with a lid that can be raised and lowered with a simple hydraulic system. # Design Variables - None # Key Concepts A hydraulic-powered machine uses pressurized water to create motion. A hinge is a movable joint that allows something to open and close. ## Prep - Build a basic example - Build just the hinge - Create a hydraulic system Notes - This project may require two classes to complete. |  | ## Materials Basic Example - 100 craft stick - 6 cube - 5 cube w/ hole - 2 cable tie - 1 dowel - 2 syringe - up to 12” tubing - hot glue Advanced Idea none |
| --- | --- | --- |

| Troubleshooting |  |
| --- | --- |
| If the box is… | Then… |
| Getting stuck in an open or closed position | The moving parts of the hinge have glue on it There is something interfering with the hinge’s movement (Ex: A craft stick glue directly across all 4 cubes with holes) |
| The hydraulic system breaks off of the box | Reinforce the cube with holes that the bottom of the syringe is tied to Make sure the cable ties are not too tight |

# Introduce the project demonstrate how it works (1 min)

# Explain how it works (2 min)

- Show students your hydraulic system and demonstrate how it works. Define hydraulics. The hydraulic system is used to raise and lower the lid.

- Show your prepared hinge and point out how it’s used in your basic example.

# Discuss how to build the box

Class 1 (10 min demo)

- Show students the project sheet and verbally outline the steps as you show them the pictures.

- Show how to efficiently create the sides of the box by laying out 13 craft sticks flat on a table in a square. Then, apply a line of glue that crosses all 13 sticks, and attach another craft stick to create a wall.

- Show how to assemble 4 sides of the box.

- Explain that the hinge is made of four cubes with holes. The two cubes on the outside of the hinge configuration are glued to the walls.

- The two cubes on the inside are glued to the lid. Therefore, it’s important to make sure that the two inside-cubes are able to move freely.

Class 2 (10-20 min demo)

- Have students build a hydraulic system step-by-step with you.

- Use an unfinished example to demonstrate how to attach the hydraulic system to the box. Have students follow along with you as you do this.

- Explain how to finish the box by gluing on the 6th wall, cover any gaps, and troubleshoot it.

  - Emphasize that many of the boxes won’t work perfectly the first time and will require troubleshooting.

- For the remainder of the class, students are challenged to elaborate on their box or embellish it.