| # Hydraulic JudoBots |  | Advanced |
| --- | --- | --- |

| # Project Goal Build a robot that’s designed to push or overturn other robots in friendly competition # Design Variables - Base shape and size - Grappling tip shape and size - Pivot column position # Key Concepts A hydraulic-powered machine uses pressurized water to create motion. ## Prep - Build a finished JudoBot - Create pairs of 12x12" squares that are 2" apart - Create a hydraulic system Notes - This project may require two or three classes to complete. - 1st class: Finish the pivot column and hinge - 2nd class: Attach the hydraulic systems, and build the base and grappling tip - 3rd class: Compete in friendly competition, troubleshoot, and redesign |  | ## Materials Basic Example - up to 40 craft stick - up to 10 cubes - 6 cube w/ hole - 2 cable tie - 1 dowel - 2 syringe - up to 12” tubing - tape - hot glue Advanced Idea none |
| --- | --- | --- |

| Troubleshooting |  |
| --- | --- |
| If the robot is… | Then… |
| Not having a full range of motion | Make sure there’s a total of 10ml of water in each hydraulic system (not including water in the tubing) Make sure there’s little to no air in the tubing Make sure the hydraulics were attached correctly and calibrated |
| - See MadeForSTEAM.com for all troubleshooting tips |  |

# Explain the premise of the project (2 min)

- Build a robot that can push other robots outside of the square, or flip them over.

- Robot’s base must stay within the 12x12" square (not including the arm).

  - Otherwise, it would be too easy to make a gigantic and impossible to move base.

- Limited to 40 craft sticks total

  - Otherwise, you could just add 200 sticks to make your robot heavier.

- Two basic strategies: push opponents outside of square, or flip them over.

# Show how it works (2 min)

- Show your example project and how it can move.

- Show students your hydraulic system and demonstrate how it works. Define hydraulics. The hydraulic system is used to raise and lower the arm, and swivel it from side to side.

# Explain how to build the JudoBot

Class 1 (5 min discussion)

- Reference the project sheet and verbalize each step to explain how to build the pivot column and hinge only; don’t try to explain every project step up front.

- Everyone will make those two parts identically.

Class 2 (20-30 min step-by-step activity and discussion)

- Have students create two hydraulic systems step-by-step with you.

- Have students attach one hydraulic system to the hinge step-by-step with you.

- Have students attach the other hydraulic system to the pivot column step-by-step with you.

- After hydraulics are attached, explain how to build the base:

  - Reminder: Must be within 12x12" square.

  - Wider bases tend to be more stable.

  - Emphasize gluing the adhesive bumpers onto the bottom of the base to increase traction so the robot doesn’t slide around the table.

- Explain the importance of the position of the pivot column:

  - Pivot column near front of the robot: If grappling tip is engaged with opponent, the robot will lean and push forward when the arm is raised; good for pushing.

  - Pivot column near back of the robot: If grappling tip is engaged with opponent, the robot will remain upright; good for lifting opponents.

- Explain how to build the arm and grappling tip:

  - Arm length matters. Longer arms can reach farther, but aren’t as strong; tend to bend more and are harder to make precise movements.

  - Two general options for the grappling tip: Something thin that can get wedged under the base to flip opponents over, or something wide to push against other opponents

Class 3 (10 min discussion)

- Explain the rules of a match:

  - Each robot is placed in a square, facing each other, with arms raised as high as possible.

  - Count down from 3 to begin.

  - The robot that remains upright or inside the square is the winner.

  - Students cannot tug on the tubing to move their robot; only use the hydraulic controls.

- A loss occurs if:

  - The robot is flipped over and can’t right itself.

  - The robot is pushed completely outside of its starting square

- A draw occurs if:

  - Neither robot can effectively reach the other.

  - Both players agree to a draw.

- Students spend the entire class time finishing their robot, participating in matches, and redesigning their robot.