| # Mechanical Hand |  | Advanced |
| --- | --- | --- |

| # Project Goal Students are challenged to build a hand that can grasp as many straws (or other material) as possible # Design Variables - Length and shape of the fingers, length of the arm, position of the trigger # Key Concepts A trigger is a device that activates a machine A hinge is a movable joint that allows something to open and close ## Prep - Build a basic example - Build just the hinge (Step 2 on project plan) - Build a separate hinge with fingers (Step 3) - Cut craft sticks in half (10 ½-pieces per student) Notes - This project may require two classes to complete. - 1st class: Build the hinge, fingers, and arm. In the second - 2nd class: Add the trigger and rubber bands. Test and redesign the hand. Bring extra materials and allow students who redesign their hand at least twice to decorate their project. |  | ## Materials Basic Example - 10 craft stick - 10 ½-stick - 8 cube - 4 cube w/ hole - 3 skewer - 2 short r.band - 1 cable tie - ½ straw - tape - caution tape Advanced Idea none |
| --- | --- | --- |

| Troubleshooting |  |
| --- | --- |
| If the hand is… | Then… |
| Getting stuck in an open or closed position | The cable tie to too tight The skewers are bent around the arm The rubber bands are missing |
| Breaking or unable to hold weight | The sticks used to build the arm are not overlapping enough The fingers are built unevenly |
| Not closing enough to grasp something | The fingers are too straight The fingers are too short Experiment! |

# Introduce the project demonstrate how it works (1 min)

Students are challenged to build a hand that can grasp as many straws (or other material) as possible.

# Explain how it works (2 min)

- The hand can open and close because the fingers are built on a hinge. Define hinge.

- Show your prepared hinge and point out how it’s used in your basic example.

- The hand is connected to a trigger with two skewers taped together. When the trigger is pulled, the hand closes.

- Point out how the rubber bands pull in hinge in the opposite direction, which automatically opens the hand.

# Show students how to build the hand (5 min)

Class 1

- Hold up your prepared hinge and explain how it’s built.

  - The straws space the fingers apart. This prevents the fingertips from colliding and preventing the hand from closing.

- Hold up your prepared hinge with fingers and explain how it’s built.

  - Emphasize the importance of angling the fingers diagonally

- Verbally explain how to build the arm by extending the back of the center finger.

  - Emphasize the importance of overlapping the sticks to create a strong beam.

Class 2

- Take the trigger off of your basic example.

- Hold up your trigger and explain how it’s built.

- Show step-by-step how to tape the two skewers together and attach the trigger.

- Show how to calibrate the trigger to fit the user’s hand.

  - Working with a partner or the teacher for this step is recommended.

# Support redesigning (2nd class) (1 min)

- Tell students that the fingers on the basic example are too short and too few to be effective. The fingers also do not have much grip.

- For this project, redesigning means adding more fingers, modifying the existing fingers, and giving the hand more grip.

  - You may hint that rubber bands may be a good material to experiment with to increase the hand’s grip.