| # Momentum Spinner |  | Intermediate |
| --- | --- | --- |

| # Project Goal Students create wind-up launchers and then design and create a set of wheels. When launched, the spinning wheels should drop from the launcher and spin rapidly in an interesting pattern for at least 5 seconds. # Design Variables - Number of wheels, wheel positions # Key Concepts Angular momentum is the tendency for a spinning object to continue to spin on its current axis. ## Prep - Cut craft sticks in half (4 per student) - Cut 24” lengths of string (1 per student) - If you have time, tie a small loop into the end of each string - Find skewers that the wheels fit snugly onto ## Notes - This project may require practice to be used effectively. - Students may have difficulty using cable ties correctly and tying the loop at the end of the string. |  | ## Materials Basic Example - 8 craft stick - 6 craft cube - 1 skewer - 2 - 4 wheels - 1 cable tie - string - tape - hot glue Advanced Ideas Racing wheels (see advanced ideas sheet) |
| --- | --- | --- |

| Troubleshooting |  |
| --- | --- |
| If the Spinning Wheels are… | Then… |
| Falling off the launcher upon release, or the string is getting jammed | Review the launching technique Make sure that there are two wheels which prevent the string from coming into contact with the launcher Make sure there is a gap at the top of the launcher where the wheels rest |

# Introduce the project and goals (2 min)

- Show students your basic example and demonstrate how it works

- Adjust your wheels and try it again. Point out how it spins differently.

- Let students know up front that this project is easy to build, but it might require some practice to use it effectively.

# Show how to build the Basic Example (6 min)

- Show step-by-step how to make the handle, launcher, and wheels.

  - Emphasize the importance of leaving a small gap on both ends of the launcher. This is necessary for the wheels to spin in place without falling off as it is being rapidly unwound.

- Make a set of wheels that looks different from your example that you used in the intro. This will encourage a variety of designs.

# Show how to use the Spinning Wheels (2 min)

- Hook the string onto the cable tie. Allow the handle to dangle from the wheels.

- Hold the wheels in both hands and turn it to wind up the string.

  - Allowing the handle to hang from the wheels will keep the string taut as it’s being wound up.

- Place the wheels onto the launcher.

- Show students how to maneuver their hands such that one hand is grasping the launcher while the other is grasping the handle.

- Hold the launcher straight up-and-down.

- Pull forcefully and directly downward on the handle to unwind the wheels.

  - The wheels may not launch correctly if the handle is pulled off to the side.

- If the wheels do not leave the launcher, then tip the launcher forward and allow the wheels to fall on the ground.

- Demonstrate this at least one more time.

# Remind students of the design variables and to focus on practice

- Remember to try changing the number and position of the wheels.

- Remind students that this project may require some practice to work correctly, and to not give up if it doesn’t seem to work the first few times.