| # Propeller Car |  | Intermediate-Advanced |
| --- | --- | --- |

| # Project Goal Students design and build propeller-powered cars that can propel itself at least 10 feet. # Design Variables - Number of rubber bands, shape of the frame, angle of the propeller shaft, axle width # Key Concepts Thrust occurs when an object pushes against the air (or other gas/fluid) to create movement Inertia is the tendency for objects to resist a change in motion Wheel Alignment means all the wheels all point in one direction ## Prep - Build a basic example - Build the 3 parts of the car individually (the frame, the wheels, and the engine) - Cut ½-sticks (1 per student) ## Notes This project is more suitable for students who have completed the helicopter, a different car project, or both. |  | ## Materials Basic Example - 10 craft stick - 1 ½-stick - 4 wheel - 2 cube - 2 skewer - 2 straw - 2 short r.band - 2 long r.band - 1 paperclip - 1 propeller - tape - hot glue Advanced Idea Returning students may use two propellers Challenge students to build a 3-wheeled car No glue variation Pyramid Propeller Car |
| --- | --- | --- |

| Troubleshooting |  |
| --- | --- |
| If the car is… | Then… |
| Not propelling itself at least 10 feet | There is not enough energy stored in the rubber band Experiment with a different combination of rubber bands Check if the wheels are turning smoothly |
| Spinning or turning | The wheels are not aligned The direction of the thrust is not aligned with the wheels Hot glue or debris is dragging on the ground |

# Introduce the project demonstrate how it works (1 min)

# Explain how it works (3 min)

- Energy is stored in the rubber bands by turning the propeller. When released, the energy from the rubber band turns the propeller, which pushes the air in one direction and pulls the car in the opposite direction. This is called thrust.

- Ask students if they think there is enough thrust to move the car if it was carrying a brick or other heavy object.

  - No, a car carrying a brick is too heavy. Heavier objects are harder to move because they have more inertia than lighter objects. Define inertia. Lighter objects usually have less inertia than heavy objects.

- The propeller car can only work if it has low inertia (light weight) since the amount of thrust it can create is limited.

# Describe how to build the project (6 min)

- Divide the project into three parts:

  - The car frame

  - The wheels

  - The engine (propeller and rubber band assembly)

- Hold up each part and verbally explain students through how to build it.

  - The basic example car frame is just a rectangle, but students can try other shapes.

  - Take apart the set of wheels and demonstrate how to assemble it.

  - Show how the engine shaft is made with three craft sticks. One craft stick is used to completely overlap the other two.

- Remind students how to attach wheels using tape in at least two places.

- Emphasize that the wheels must be aligned, or all ‘pointing’ in the same direction/ Otherwise the car will turn. Students who want turning cars can purposely misalign the wheels.

- Finally, show students that the propeller must be kept away from the ground.

# Support redesigning (2 min)

- Tell students that the number and type of rubber bands is important to getting their car to create enough thrust. Suggest they try different combinations of short and long rubber bands.

- Remember that the length of the skewer can be adjusted for a narrower set of wheels.

- Students can also experiment with different shaped car frames.