| # Slingshot Car |  | Beginner - Intermediate |
| --- | --- | --- |

| # Project Goal Students build cars that are rapidly propelled across the floor at least 10 feet by a simple slingshot. # Design Variables - Axle width, number of wheels, wheel alignment # Key Concepts Wheel alignment means all the wheels all point in one direction Energy transformation is when one kind of energy turns into a different form ## Prep Bend paperclips into hooks (1 per student) Set up glue gun (for lecture) ## Safety Launch cars only horizontally across the floor. Designate a launching area and direction when students begin to test. |  | ## Materials Basic Example - 8 craft stick - 2 skewer - 2 straw - up to 6 wheels - 1 long r.band - tape - hot glue (1 per 2 students) - pencil Advanced Ideas Challenge students to build a driver’s seat for their egg - craft cube - cup - plastic egg |
| --- | --- | --- |

| Troubleshooting |  |
| --- | --- |
| If the car… | Then… |
| Spinning out of control | Check the wheel alignment. The axles should be parallel with each other and perpendicular to the direction of the car Hold the very back-center of the car before releasing |
| Not launching successfully | Launch with the slingshot perpendicular to the ground Check if the slingshot is twisted Review launching technique |

# Introduce project and demonstrate how it works (1 min)

# Demonstrate how to build the basic example (6 min)

- Show step-by-step how to make the basic example.

- For an intermediate class, you can pre-make the wheels and car frame to speed up the lecture.

- Emphasize the importance of using both tape and glue to attach the paperclip hook.

# Demonstrate how to launch the car (2 min)

- Place the car in the designated testing area.

- Firmly grasp the slingshot with the rubber band side pointed straight down.

- Drape the rubber band onto the paperclip, and then pull the rubber band into the hook.

- Hold the very back center of the car. Pull the car back while keeping it level with the ground, and then release.