| # Straw Rocket |  | Beginner - Advanced |
| --- | --- | --- |

| # Project Goal Students design and build small rockets that are launched at least 20 feet from a simple slingshot. # Design Variables - Straw length, number of fins, shape and size of fins, fin position - Optional: Weight at the tip of the rocket # Key Concepts Drag occurs when air (or other gas/fluid) pushes back against a moving object and slows it down. Fins are flat things attached to rockets that help maintain a particular direction. ## Prep - Bend paperclips (1 per student) ## Safety When indoors, designate a target and have students take turns Always aim rockets above other people’s heads. |  | ## Materials Basic Example - 1 straw - 1 paperclip - 1 craft stick - 1 long r.band - ⅛ piece of cardstock - tape Advanced Ideas Challenge students to build a better slingshot and/or a longer rocket - craft cube - short r.band - extra straw - hot glue stick (for adding weight to the tip) - hot glue (for building slingshot) |
| --- | --- | --- |

| Troubleshooting |  |
| --- | --- |
| If the rocket is… | Then… |
| Getting stuck on the rubber band or colliding with the slingshot | Check if the rubber band is twisted Load the rocket with the hook pointed downward Hold the slingshot straight up and down Flick the slingshot forward as you release the rocket |
| Not aerodynamic (suddenly slowing mid-flight, tumbling, or not flying very far) | Check if the fins are straight and secured (not wobbly) Check for excess tape Fins are very uneven in shape, size, and/or placement Remember to hold the very back of the rocket |

# Introduce the project and demonstrate your example (1 min)

- Showing students how the rocket works will help get their attention and build up enthusiasm for the project.

# Explain how it works using key concepts (2 min)

- Tell students that the fins are what make the rocket go in a straight path.

- Fins are not wings. Wings help lift aircraft up. Fins don’t help the rocket fly up. Fins keep the rocket going in the direction that the fins are pointed.

  - Think of it like this: If a flying rocket begins to turn, the broad side of the fins becomes more exposed to the oncoming air. This creates drag, which pushes on the broad side of the fins until the rocket straightens out.

# Build a basic example (4 min)

- Show step-by-step how to build a basic example.

- Verbalize what you’re building as you do so.

- Cut a variety of different fin shapes and sizes. This will encourage varied designs.

- Emphasize the importance of having straight and secure fins.

# Designate the safe testing area and demonstrate how to launch the rocket (3 min)

- Demonstrate the rocket launch at your testing area and outline the testing rules.

- Firmly grasp the slingshot and hold it at a right angle to your forearm.

- Hold the very back of the rocket (behind the fins).

- Load the rocket onto the slingshot with the hook pointed downward.

- Pull back and release

- Allow the slingshot to tip forward upon release. This will help prevent the fins from colliding with the slingshot.

- Repeat your demonstration at least one more time.