| # Trigger Launcher |  | Beginner - Advanced |
| --- | --- | --- |

| # Project Goal Build and modify a simple trigger-release launcher that shoots at least 10 feet. # Design Variables - Number of rubber bands, length of the launcher, number of combined launchers # Key Concepts Trajectory is the path that an object follows through the air or other fluid. Trigger is a device that sets off a mechanism. Energy transformation is when one kind of energy turns into a different form. ## Prep Carefully cut corks into quarters (2 per student) ## Safety Always aim launchers away from people, even when unloaded. Launch only cork disks. |  | ## Materials Basic Example - 6 craft stick - 3 craft cube - 1 short r.band - 1 clothespin - ¼ cork - tape - hot glue - caution tape Advanced Ideas Trigger Launcher Car - 4 wheel - 2 skewer Hinged Launcher - 3 cube w/ hole - 1 skewer - extra materials |
| --- | --- | --- |

| Troubleshooting |  |
| --- | --- |
| If the cork is… | Then… |
| Launching less than 10 feet | The cork is not properly loaded The craft sticks are not ‘stair-stepping’ down from the trigger Use a short rubber band |
| Launching in an unpredictable or unstable trajectory | The cork is not loaded properly The craft sticks are not ‘stair-stepping’ down from the trigger Something is interfering with the cork when launched |

# Introduce project name and demonstrate how it works (1 min)

- Establish safety protocol at the beginning of class

# Explain how it works (2 min)

- Energy is stored in the rubber bands by stretching them.

- The rubber bands are held in place with a clothespin, which acts as a trigger.

- A cork disk is placed between the rubber band strands. This holds the cork piece in place and also ensures that the rubber band catches it upon being released.

- Pushing the trigger releases the rubber bands and the stored energy transforms into moving energy (kinetic energy) by launching the cork disk.

- Define trajectory. The user can change the cork’s trajectory by changing the direction the cork shooter is pointed. Aiming higher will create a long arc-like trajectory, and aiming it straight will create a flatter trajectory.

# Explain how to use it (2 min)

- Use one hand to grasp the launcher and also open the trigger.

- Use your index and middle fingers from the other hand to stretch the rubber band.

- Insert the section of rubber band that is between the fingers into the clothespin.

  - Attempting to load the launcher with just one finger will result in that finger getting stuck in the clothespin.

- Place the cork disk between the rubber band strands. Aim away from people and fire!

# Show students how to build the Basic Example (5 min)

- Quickly demonstrate how to build a trigger launcher from start to finish.

# Support redesigning (1 min)

- Tell students that this project is pretty simple, so they should think of ways to make their design better than yours.

- Name some suggestions such as making a double or triple launcher, or adding more rubber bands, or making their launcher twice as long.