| # Zip Line Racer |  | Intermediate |
| --- | --- | --- |

| # Project Goal Create a propeller-powered vehicle that rapidly accelerates along a zip line, and travels at least 15'. # Design Variables - Number of rubber bands, shape of the paper cutout. # Key Concepts Thrust occurs when an object pushes against the air (or other gas/fluid) to create movement. Torque is a twisting force that tends to cause rotation. ## Prep - Build a basic example. - Tightly stretch the mason line between 2 fixed points in the room. Make it at least 20' long. Set it up low enough so students can reach it, but high enough so they must stand on a chair to launch their racer. This helps prevent students from accidentally being struck by a racer. - (Optional) Make cardboard templates of animals, race cars, etc. ## Notes The launching technique of this project can be tricky. Be sure to coach students during the first time each person tests. |  | ## Materials Basic Example - 7 craft stick - 2 long r.band - 3 paperclip - 1 propeller - Nylon mason line - tape - hot glue Advanced Idea Returning students may use two propellers; challenge them to build an extra-fast racer |
| --- | --- | --- |

| Troubleshooting |  |
| --- | --- |
| If the racer is… | Then… |
| Not propelling itself at least 15 feet | There is not enough energy stored in the rubber band Experiment with a different combination of rubber bands |
| Falling off the zip line | Do not pull down on the racer when it’s on the zip line Try winding up the rubber bands less Attach a larger paper cutout |

Introduce the project demonstrate how it works (1 min)

# Explain how it works (3 min)

- Energy is stored in the rubber bands. When released, the energy from the rubber band turns the propeller, which pushes the air in one direction and pulls the racer in the opposite direction. This is called thrust.

- The energy in the rubber bands is not only turning the propeller. Since the rubber bands are also attached to the back of the racer, the entire frame of the racer is being rotated, just like the propeller. This rotational force that’s twisting the propeller and the frame is called torque.

- With enough toque – which is generated from the twisting force of the rubber bands – the entire racer could flip upside down and crash. To counteract this, a paper cutout is attached to the frame.

- The paper cutout prevents the racer from flipping over. If the racer starts to flip over, the paper gets pushed against the air, and makes it too difficult for the twisting force to flip the racer over.

# Describe how to build the project (6 min)

- Quickly build the project step-by-step.

- Emphasize that the paperclips on top of the racer must be turned so the U-bend rests on the zip line without causing excessive friction.

- (Optional: If you created templates, then explain how to trace and cut out shapes).

# Show how to safely launch the racers (2 min)

- Face the top of the propeller toward you and turn clockwise until the rubber band becomes significantly more difficult to wind (50-80 rotations).

  - You can also look at the rubber band as it gets twisted. Once the entire rubber band begins coiled around itself three times then it has enough energy (try winding the racer and watch the rubber band).

- Gently rest the racer’s paperclips onto the zip line. Do not pull down on the zip line! If you do, then when you let go of the racer, it will bounce up and immediately fall off, which can be dangerous.

- Maneuver your hands so you’re only holding onto the propeller. Quickly let go!

# Support redesigning (2 min)

- Tell students that the number and type of rubber bands is important to getting their racer to create enough thrust. Suggest they try different combinations of short and long rubber bands.

- Remember that the paper cutout has a big effect on how much energy can be used to launch the racer before it’s at risk of flipping over.

- Students can also experiment with different shaped frames.