STEM  INVENTIONS

BOW AND ARROW                                           Beginner to Advanced

| ▍ PROJECT GOAL Students build truss-based bow frames that launch weighted straws at least 15 feet. ▍ DESIGN VARIABLES Amount of weight in arrow tip, length of arrow, size of bow frame, number of rubber bands ▍ KEY CONCEPTS - Trusses are triangles used in engineering to strengthen a structure. - Energy transformation is when one kind of energy turns into a different form. - Momentum is the force that causes a moving object to continue to move in the direction it is already going toward ▍ PREP - Cut glue sticks into lengths between ½” and 2” (3 pieces per student). - Set up a designated testing area with a target. | ▍ SAFETY Always aim the bow away from people and breakables, even when unloaded. |
| --- | --- |

MATERIALS

Basic Example

- 12	craft sticks

- 2	½-sticks

- 5	craft cubes

- 1 short rubber band

- 3 straws

- pom pom

- tape

- hot glue

- caution tape

ADVANCED IDEA

Challenge students to build a crossbow.

- clothespin

- long rubber band

Troubleshooting

| IF THE ARROW IS… | THEN… |
| --- | --- |
| Firing less than 15 ft | - There is not enough energy stored in the rubber band - The bow string is being grasped instead of pinched |
| Launching in an unstable trajectory | - The arrow is loaded perpendicular to the rubber band - The user’s hand is interfering with the arrow on launch |
| Getting stuck between the r. band and frame | - The arrow is too short - The bowstring is being drawn back too far |
| Launched properly but still falling short | - Check if there is a piece of glue stick at the tip - Check for interfering materials (Ex: loose tape) |
| Breaking or splintering | - Carefully wrap the arrow shaft in tape. |

STEM  INVENTIONS

Sample Lesson Guide

| 1 | Introduce project name and demonstrate how it works 1 min |
| --- | --- |

- Establish the safety protocol at the beginning of class.

| 2 | Explain how it works 2 min |
| --- | --- |

- Show that the Basic Example is made of many triangles, or trusses. Define truss.

- Use a square and a triangle (or draw on the board) to demonstrate why trusses are strong.

- Energy is stored in the rubber bands by stretching them.

- When released, the stored energy transforms into moving energy (kinetic energy) by firing the arrow.

- The piece of glue stick at the tip of the arrow adds weight to it. When fired, that weight carries momentum.

- Think of it like this: Throwing an empty plastic bag won’t make it go very far. It is not heavy enough to push through the air around it. If you fill the bag with chips, then it has more weight. That extra weight can push through the air more easily.

| 3 | Explain how to use it 2 min |
| --- | --- |

- Load the arrow through the front of the bow and place the notch over the rubber band.

- Pinch the notch, but do not grasp the rubber band.

- Grasp the frame of the bow with the other hand, aim safely, and fire.

| 4 | Show students how to build the Basic Example 5 min |
| --- | --- |

- Show students the Project Sheet and walk them through the steps of construction.

- If you have a younger class, then build a bow frame step-by-step in front of them.

- Show students how to make an arrow.

- Demonstrate how to insert a piece of glue stick into one end of the arrow and tape it in place.

- Demonstrate how to wrap tape around the other end and then use scissors to cut out the notch.

- You may need to cut the notch for the students.

| 5 | Support redesigning 1 min |
| --- | --- |

- Tell students that this project can be changed by adding more or less weight to the tip, making the bow bigger or smaller, adding more rubber bands, or perhaps finding a totally new shape for the bow frame.