STEM  INVENTIONS

HYDRAULIC HAND                                      Advanced

| ▍ PROJECT GOAL Students are challenged to build a hand that can grasp as many different items as possible ▍ DESIGN VARIABLES Length and shape of the fingers, strength of the arm ▍ KEY CONCEPTS - A hydraulic-powered machine uses pressurized water to create motion - A hinge is a movable joint that allows something to open and close ▍ PREP - Build a basic example - Build just the hinge - Assemble a hydraulic system - Cut craft sticks in half (12 ½-pieces per student) - Gather a variety of smallish items to pick up |  |
| --- | --- |

MATERIALS

Basic Example

- 13	craft stick

- 12	½-stick

- 11	cube

- 3	cube w/ hole

- 1	skewer

- 2	syringe

- 6" tubing

- 1	cable tie

- ½ straw

- tape

- hot glue

ADVANCED IDEA

none

▍ NOTES

This project may require two classes to complete.

1st class: Build the hinge, fingers, and arm. In the second

2nd class: Add the hydraulics. Test and redesign the hand. Bring extra materials and allow students who redesign their hand at least twice

Troubleshooting

| IF THE HAND IS … | THEN… |
| --- | --- |
| Getting stuck in an open or closed position | - The cable tie to too tight - There is something interfering with the hinge’s movement (Ex: A craft stick glue directly across all4 cubes with holes) - The hydraulic system doesn’t have the right amount of water in it |
| Not closing enough to grasp something | - The fingers are too straight - The fingers are too short - Experiment! |

STEM  INVENTIONS

Sample Lesson Guide

| 1 | Introduce the project demonstrate how it works 1 min |
| --- | --- |

Students are challenged to build a hand that can grasp as different items as possible

| 2 | Explain how it works 2 min |
| --- | --- |

- The hand can open and close because the fingers are built on a hinge. Define hinge.

- Show your prepared hinge and point out how it’s used in your basic example.

- The hand is connected to a hydraulic system. Show students your hydraulic system and demonstrate how it works. Define hydraulics. The hydraulic system is used to open and close the hand.

- Point out how your basic example can pick up some things, but there is room for improvement.

| 3 | Show students how to build the hand 10 min |
| --- | --- |

Class 1

- Hold up your prepared hinge and explain how it’s built.

- The straws space the fingers apart. This prevents the fingertips from colliding and preventing the hand from closing.

- Hold up your prepared hinge with fingers and explain how it’s built.

- Emphasize the importance of angling the fingers diagonally

- Verbally explain how to build the arm by extending the back of the center finger.

- Emphasize the importance of overlapping the sticks to create a strong beam.

Class 2

- Have students build a hydraulic system step-by-step with you.

- Use an unfinished example to demonstrate how to attach the hydraulic system to the hand. Have students follow along with you as you do this.

- For the remainder of the class, students are challenged to redesign their hand to pick up as many items as possible.

| 4 | Support redesigning (2nd class) 1 min |
| --- | --- |

- Tell students that the fingers on the basic example are too short and too few to be effective. The fingers also do not have much grip.

- For this project, redesigning means adding more fingers, modifying the existing fingers, and giving the hand more grip.

- You may hint that rubber bands may be a good material to experiment with to increase the hand’s grip.