STEM  INVENTIONS

HYDRAULIC JUDOBOT                                                 Advanced

| ▍ PROJECT GOAL Build a robot that’s designed to push or overturn other robots in friendly competition ▍ DESIGN VARIABLES Base shape and size Grappling tip shape and size Pivot column position ▍ KEY CONCEPTS - A hydraulic-powered machine uses pressurized water to create motion. ▍ PREP - Build a finished JudoBot - Create pairs of 12x12" squares that are 2" apart - Create a hydraulic system |  |
| --- | --- |

MATERIALS

Basic Example

- up to 40	craft stick

- up to 10	cubes

- 6	cube w/ hole

- 2	cable tie

- 1	dowel

- 2	syringe

- up to 12” tubing

- tape

- hot glue

ADVANCED IDEA

none

▍ NOTES

This project may require two or three classes to complete.

1st class: Finish the pivot column and hinge

2nd class: Attach the hydraulic systems, and build the base and grappling tip

3rd class: Compete in friendly competition, troubleshoot, and redesign

Troubleshooting

| IF THE ROBOT IS… | THEN… |
| --- | --- |
| Not having a full range of motion | - Make sure there’s a total of10ml of water in each hydraulic system (not including water in the tubing) - Make sure there’s little to no air in the tubing - Make sure the hydraulics were attached correctly and calibrated |

See MadeForSTEAM.com for all troubleshooting tips

STEM  INVENTIONS

Sample Lesson Guide

| 1 | Explain the premise of the project 2 min |
| --- | --- |

- Build a robot that can push other robots outside of the square, or flip them over.

- Robot’s base must stay within the 12x12" square (not including the arm).

- Otherwise, it would be too easy to make a gigantic and impossible to move base.

- Limited to 40 craft sticks total

- Otherwise, you could just add 200 sticks to make your robot heavier.

- Two basic strategies: push opponents outside of square, or flip them over.

| 2 | Show how it works 2 min |
| --- | --- |

- Show your example project and how it can move.

- Show students your hydraulic system and demonstrate how it works. Define hydraulics. The hydraulic system is used to raise and lower the arm, and swivel it from side to side.

| 3 | Explain how to build the JudoBot |
| --- | --- |

| 3.1 | Class 1 5 min discussion |
| --- | --- |

- Reference the project sheet and verbalize each step to explain how to build the pivot column and hinge only; don’t try to explain every project step up front.

- Everyone will make those two parts identically.

| 3.2 | Class 2 20-30 min step-by-step activity and discussion |
| --- | --- |

- Have students create two hydraulic systems step-by-step with you.

- Have students attach one hydraulic system to the hinge step-by-step with you.

- Have students attach the other hydraulic system to the pivot column step-by-step with you.

- After hydraulics are attached, explain how to build the base:

- Reminder: Must be within 12x12" square.

- Wider bases tend to be more stable.

- Emphasize gluing the adhesive bumpers onto the bottom of the base to increase traction so the robot doesn’t slide around the table.

- Explain the importance of the position of the pivot column:

- Pivot column near front of the robot: If grappling tip is engaged with opponent, the robot will lean and push forward when the arm is raised; good for pushing.

- Pivot column near back of the robot: If grappling tip is engaged with opponent, the robot will remain upright; good for lifting opponents.

- Explain how to build the arm and grappling tip:

- Arm length matters. Longer arms can reach farther, but aren’t as strong; tend to bend more and are harder to make precise movements.

- Two general options for the grappling tip: Something thin that can get wedged under the base to flip opponents over, or something wide to push against other opponents

| 3.3 | Class 3 10 min discussion |
| --- | --- |

- Explain the rules of a match:

- Each robot is placed in a square, facing each other, with arms raised as high as possible.

- Count down from 3 to begin.

- The robot that remains upright or inside the square is the winner.

- Students cannot tug on the tubing to move their robot; only use the hydraulic controls.

- A loss occurs if:

- The robot is flipped over and can’t right itself.

- The robot is pushed completely outside of its starting square

- A draw occurs if:

- Neither robot can effectively reach the other.

- Both players agree to a draw.

- Students spend the entire class time finishing their robot, participating in matches, and redesigning their robot.