STEM  INVENTIONS

HYDRAULIC/PNEUMATIC MACHINE              Beginner to Advanced

| ▍ PROJECT GOAL Build a machine that’s powered by a simple hydraulic or pneumatic system. The hinge forms the foundation for any project of the student’s imagination. ▍ DESIGN VARIABLES This is an open-ended project. Once the hinge is complete, students will decide what to transform it into. ▍ KEY CONCEPTS - A hydraulic-powered machine uses pressurized liquid to create motion. - A pneumatic-powered machine uses pressurized air to create motion. ▍ PREP - Build a basic example - Create a pneumatic system | ▍ NOTES You may choose to make the hydraulic or pneumatic powered. Hydraulics are stronger, but slightly more challenging to create. |
| --- | --- |

MATERIALS

Basic Example

- 10	craft stick

- 2	cube

- 3	cube w/ hole

- (or syringe adapter)

- 1	cable tie

- 2	syringe

- About 18" tubing

- tape

- hot glue

ADVANCED IDEA

none

Troubleshooting

| IF THE HINGE IS… | THEN… |
| --- | --- |
| Not moving easily | - The cable tie to too tight - The hinge has glue on it - The output syringe is rubbing against something or getting caught on the edge of a craft stick |
| Not raising or lowering the arm all the way | - There is too much or too little water/air in the system |

STEM  INVENTIONS

Sample Lesson Guide

| 1 | Introduce the project demonstrate how it works 1 min |
| --- | --- |

- Explain that the hinge is not the finished project. Instead, students use it as a foundation for a machine of their choice.

| 2 | Explain how it works 2 min |
| --- | --- |

- Show students your hydraulic or pneumatic system and demonstrate how it works. Define hydraulics or pneumatics.

- The hydraulic/pneumatic system is used to raise and lower the hinge arm.

| 3 | Demonstrate how to build the hinge 6 min |
| --- | --- |

- Show step-by-step how to build the hinge. For advanced classes, you may verbally outline how to build the hinge while referencing the project sheet.

- Show step-by-step how to build the hydraulic or pneumatic system.

- Emphasize the importance of having just one syringe filled with water/air and having the other empty.

| 4 | Brainstorm ideas 2 min |
| --- | --- |

- Explain that the hinge is just a starting place for more a more interesting project.

- Orient the hinge in different directions and suggest some ways that the hinge can be transformed into a much more interesting project. For example:

- Turned on it’s side, the can be used as an automatic house door.

- Lying flat, it can be used as a roof for a convertible car.

- Lying flat, it can be used as a bulldozer or giant hammer.

- Standing upright, it can become a crane.

- A cork launcher can be mounted on the hinge.

- Emphasize that it’s up to the students to decide what to turn their hinge into.