STEM  INVENTIONS

MECHANICAL HAND                                                     Advanced

| ▍ PROJECT GOAL Students are challenged to build a hand that can grasp as many straws (or other material) as possible ▍ DESIGN VARIABLES Length and shape of the fingers, length of the arm, position of the trigger ▍ KEY CONCEPTS - A trigger is a device that activates a machine - A hinge is a movable joint that allows something to open and close ▍ PREP - Build a basic example - Build just the hinge (Step 2 on project plan) - Build a separate hinge with fingers (Step 3) - Cut craft sticks in half (10 ½-pieces per student) |  |
| --- | --- |

MATERIALS

Basic Example

- 10	craft stick

- 10	½-stick

- 8	cube

- 4	cube w/ hole

- 3	skewer

- 2	short rubber band

- 1	cable tie

- ½ straw

- tape

- caution tape

ADVANCED IDEA

none

▍ NOTES

This project may require two classes to complete.

1st class: Build the hinge, fingers, and arm. In the second

2nd class: Add the trigger and rubber bands. Test and redesign the hand. Bring extra materials and allow students who redesign their hand at least twice to decorate their project.

Troubleshooting

| IF THE HAND IS… | THEN… |
| --- | --- |
| Getting stuck in an open or closed position | - The cable tie to too tight - The skewers are bent around the arm - The rubber bands are missing |
| Breaking or unable to hold weight | - The sticks used to build the arm are not overlapping enough - The fingers are built unevenly |
| Not closing enough to grasp something | - The fingers are too straight - The fingers are too short - Experiment! |

STEM  INVENTIONS

Sample Lesson Guide

| 1 | Introduce the project demonstrate how it works 1 min |
| --- | --- |

Students are challenged to build a hand that can grasp as many straws (or other material) as possible.

| 2 | Explain how it works 2 min |
| --- | --- |

- The hand can open and close because the fingers are built on a hinge. Define hinge.

- Show your prepared hinge and point out how it’s used in your basic example.

- The hand is connected to a trigger with two skewers taped together. When the trigger is pulled, the hand closes.

- Point out how the rubber bands pull in hinge in the opposite direction, which automatically opens the hand.

| 3 | Show students how to build the hand 5 min |
| --- | --- |

Class 1

- Hold up your prepared hinge and explain how it’s built.

- The straws space the fingers apart. This prevents the fingertips from colliding and preventing the hand from closing.

- Hold up your prepared hinge with fingers and explain how it’s built.

- Emphasize the importance of angling the fingers diagonally

- Verbally explain how to build the arm by extending the back of the center finger.

- Emphasize the importance of overlapping the sticks to create a strong beam.

Class 2

- Take the trigger off of your basic example.

- Hold up your trigger and explain how it’s built.

- Show step-by-step how to tape the two skewers together and attach the trigger.

- Show how to calibrate the trigger to fit the user’s hand.

- Working with a partner or the teacher for this step is recommended.

| 4 | Support redesigning (2nd class) 1 min |
| --- | --- |

- Tell students that the fingers on the basic example are too short and too few to be effective. The fingers also do not have much grip.

- For this project, redesigning means adding more fingers, modifying the existing fingers, and giving the hand more grip.

- You may hint that rubber bands may be a good material to experiment with to increase the hand’s grip.