STEM  INVENTIONS

MOMENTUM SPINNER                                               Intermediate

| ▍ PROJECT GOAL Students create wind-up launchers and then design and create a set of wheels. When launched, the spinning wheels should drop from the launcher and spin rapidly in an interesting pattern for at least 5 seconds. ▍ DESIGN VARIABLES Number of wheels, wheel positions ▍ KEY CONCEPTS - Angular momentum is the tendency for a spinning object to continue to spin on its current axis. ▍ PREP - Cut craft sticks in half (4 per student) - Cut 24” lengths of string (1 per student) - If you have time, tie a small loop into the end of each string - Find skewers that the wheels fit snugly onto | ▍ NOTES This project may require practice to be used effectively. Students may have difficulty using cable ties correctly and tying the loop at the end of the string. |
| --- | --- |

MATERIALS

Basic Example

- 8	craft stick

- 6	craft cube

- 1	skewer

- 2	- 4 wheels

- 1	cable tie

- string

- tape

- hot glue

ADVANCED IDEAS

Racing wheels (see advanced ideas sheet)

Troubleshooting

| IF THE SPINNING WHEELS ARE… | THEN… |
| --- | --- |
| Falling off the launcher upon release, or the string is getting jammed | - Review the launching technique - Make sure that there are two wheels which prevent the string from coming into contact with the launcher - Make sure there is a gap at the top of the launcher where the wheels rest |

STEM  INVENTIONS

Sample Lesson Guide

| 1 | Introduce the project and goals 2 min |
| --- | --- |

- Show students your basic example and demonstrate how it works

- Adjust your wheels and try it again. Point out how it spins differently.

- Let students know up front that this project is easy to build, but it might require some practice to use it effectively.

| 2 | Show how to build the Basic Example 6 min |
| --- | --- |

- Show step-by-step how to make the handle, launcher, and wheels.

- Emphasize the importance of leaving a small gap on both ends of the launcher. This is necessary for the wheels to spin in place without falling off as it is being rapidly unwound.

- Make a set of wheels that looks different from your example that you used in the intro. This will encourage a variety of designs.

| 3 | Show how to use the Spinning Wheels 2 min |
| --- | --- |

- Hook the string onto the cable tie. Allow the handle to dangle from the wheels.

- Hold the wheels in both hands and turn it to wind up the string.

- Allowing the handle to hang from the wheels will keep the string taut as it’s being wound up.

- Place the wheels onto the launcher.

- Show students how to maneuver their hands such that one hand is grasping the launcher while the other is grasping the handle.

- Hold the launcher straight up-and-down.

- Pull forcefully and directly downward on the handle to unwind the wheels.

- The wheels may not launch correctly if the handle is pulled off to the side.

- If the wheels do not leave the launcher, then tip the launcher forward and allow the wheels to fall on the ground.

- Demonstrate this at least one more time.

| 4 | Remind students of the design variables and to focus on practice |
| --- | --- |

- Remember to try changing the number and position of the wheels.

- Remind students that this project may require some practice to work correctly, and to not give up if it doesn’t seem to work the first few times.