STEM  INVENTIONS

PNEUMATIC MACHINE                                               Intermediate

| ▍ PROJECT GOAL Students create identical pneumatic machines and then use the remaining class time to customize the machine’s function. ▍ DESIGN VARIABLES Length and function of pneumatic arm, open-ended additions to improve stability, function, and style ▍ KEY CONCEPTS - Pneumatic machines use compressed air to move. ▍ PREP - Cut tubing to 6” lengths (1 per student) - Cut straws into quarters (6 per student) - Cut straws into eighths (2 per student) - Optional for young classes: create the pneumatic systems (1 per student) |  |
| --- | --- |

MATERIALS

Basic Example

- 9	craft stick

- 2	jumbo stick

- 2	straw

- 2	syringe

- 1-2 pipe cleaner

- tubing

- tape

ADVANCED IDEAS

Challenge students to turn the pneumatic arm into a cork launcher.

Add wheels onto the machine.

Bring hot glue.

- clothespin

- cork

- sm. rubber band

- skewer

- wheel

- hot glue

▍ NOTES

This project should be taught in two parts:

Create the triangular frame

Create and attach the pneumatic system, then customize the machine

Troubleshooting

| IF THE PNEUMATIC ARM IS… | THEN… |
| --- | --- |
| Bending sideways | - Tighten the pipe cleaner that connects the two jumbo sticks - Tape the syringe more firmly to the jumbo stick - Check to make sure the tubing is taped to the frame |
| Not lifting or lowering completely | - The pneumatic system has too much or too little air in it. - The pipe cleaner that is used to connect the syringe to the jumbo stick is loose, not attached properly, or attached in the wrong place. |

STEM  INVENTIONS

Sample Lesson Guide

| 1 | Introduce the project and goals 1 min |
| --- | --- |

- Show students your basic example and explain how it works.

- Explain that this class will be split into two parts: 1. Create the triangular frame. 2. Create and attach the pneumatic system, then customize the machine.

| 2 | Show how to create the triangular frame 2 min |
| --- | --- |

- Students should be familiar with the stick-and-straw building technique for this lesson.

- Show how to connect two triangles to form a prism.

- Allow students to gather 9 sticks and 6 straw pieces and build the frame.

- Students who finish early can help other classmates finish.

- You may give hands-on help to students who are having a hard time keeping up with the class on this step.

- Once all students have built the frame, move onto the next part.

| 3 | Show how to create the pneumatic arm 5 min |
| --- | --- |

- Show step-by-step how to create the pneumatic arm.

- Show how to create the pneumatic system if you did not prep it.

- Tape an eighth-straw piece onto the end of two jumbo sticks.

- Use a pipe cleaner to tie the sticks together through the straw pieces. Twist it tightly.

- Tape the syringe near one end of the jumbo stick.

- Use a pipe cleaner to tie the other end of the piston to the jumbo stick. Twist it tightly.

- Tape the assembly onto the triangular frame.

- Tape the tubing to the frame.

| 4 | Support diverse designs |
| --- | --- |

- Explain that the sample you just built is not finished. It’s up to the students to make it do something useful.

- Give students a few ideas such as extending the arm, adding onto the base to make it more stable, adding something to the end of the arm.

- Introduce the Advanced Ideas if you brought the materials.

- Remind students one more time of the steps to create and attach the pneumatic arm before they begin building.