STEM  INVENTIONS

SLINGSHOT CAR                                       Beginner to Intermediate

| ▍ PROJECT GOAL Students build cars that are rapidly propelled across the floor at least 10 feet by a simple slingshot. ▍ DESIGN VARIABLES Axle width, number of wheels, wheel alignment ▍ KEY CONCEPTS - Wheel alignment means all the wheels all point in one direction - Energy transformation is when one kind of energy turns into a different form ▍ PREP - Bend paperclips into hooks (1 per student) - Set up glue gun (for lecture) |  |
| --- | --- |

MATERIALS

Basic Example

- 8	craft stick

- 2	skewer

- 2	straw

- up to 6	wheels

- 1	long rubber band

- tape

- hot glue (1 per 2 students)

- pencil

ADVANCED IDEAS

Challenge students to build a driver’s seat for their egg

- craft cube

- cup

- plastic egg

▍ SAFETY

Launch cars only horizontally across the floor. Designate a launching area and direction when students begin to test.

Troubleshooting

| IF THE CAR… | THEN… |
| --- | --- |
| Spinning out of control | - Check the wheel alignment. The axles should be parallel with each other and perpendicular to the direction of the car - Hold the very back-center of the car before releasing |
| Not launching successfully | - Launch with the slingshot perpendicular to the ground - Check if the slingshot is twisted - Review launching technique |

STEM  INVENTIONS

Sample Lesson Guide

| 1 | Introduce project and demonstrate how it works 1 min |
| --- | --- |

| 2 | Demonstrate how to build the basic example 6 min |
| --- | --- |

- Show step-by-step how to make the basic example.

- For an intermediate class, you can pre-make the wheels and car frame to speed up the lecture.

- Emphasize the importance of using both tape and glue to attach the paperclip hook.

| 3 | Demonstrate how to launch the car 2 min |
| --- | --- |

- Place the car in the designated testing area.

- Firmly grasp the slingshot with the rubber band side pointed straight down.

- Drape the rubber band onto the paperclip, and then pull the rubber band into the hook.

- Hold the very back center of the car. Pull the car back while keeping it level with the ground, and then release.